Title: Object Detection with YOLO

## 1. Dataset Description and Preprocessing Steps

After carefully curating a dataset of 1,000 street images featuring traffic signs, traffic lights, and various vehicles, the images were split into 640 for training, 180 for validation, and 180 for testing. This balanced ratio provides sufficient data for model learning while leaving enough unseen examples to validate and test performance.

### Preprocessing Steps:

- Annotation:

  - Images were labeled using Roboflow’s auto-annotation tool, with classes for traffic signs (stop_sign, yield_sign, speed_limit_sign, no_entry_sign, etc.), vehicles (car, bus, truck, motorcycle, bicycle), and pedestrians.

- Data Augmentation:

  - To improve small object detection, several techniques were applied:

    - Crop (random cropping ~70–90% of the image area) to handle object scale variation.

    - Rotation (±15°) and Shear (±10°) to simulate tilting, angles, and perspective changes.

    - Mosaic augmentation to combine multiple images, giving more context and multi-scale examples.

- Image Resolution:

  - The training pipeline used an input size of 1280×1280 for images. This higher resolution ensures more pixel detail for small and distant objects (e.g., traffic signs seen from afar).

- Dataset Splitting:

  - Train (640 images): Used for the main learning phase.

  - Validation (180 images): Monitored during training for early stopping and hyperparameter tuning.

  - Test (180 images): Provided a final, unbiased assessment of model performance.

This preprocessing strategy aims to maximize the model’s ability to recognize small, distant targets in complex urban environments.

## 2. Challenges Faced in Detecting Small Objects

- Limited Pixel Information:

  - Distant objects (like overhead traffic signs) occupy only a handful of pixels, making feature extraction difficult.

- Busy Backgrounds:

  - Urban scenes with vehicles, pedestrians, buildings, and signage create clutter, causing false positives and missed detections for tiny signs.

- Occlusion:

  - Vehicles or other infrastructure can partially block signs and lights, reducing the available features for detection.

- Varying Lighting and Angles:

  - Early morning/late evening glare, harsh midday sun, or angled camera views add inconsistencies that complicate detection.

Despite these obstacles, targeted preprocessing and careful augmentation help mitigate many of these issues, prompting iterative improvements in model performance.

## 3. Experimentation Results with Different Settings

Following the basic training run (YOLOv8, 640×640 resolution), a series of experiments were conducted:

- Higher Resolution (1280×1280):

  - Improved recall on small signs by capturing more detail.

  - Slight increase in training time and GPU memory usage, but delivered more accurate bounding boxes for distant objects.

- Data Augmentations:

  - Crop and Rotation: Increased the model’s robustness to scale and orientation variations, raising precision for overhead signs.

  - Shear: Helped detect angled or partially visible signs around corners.

  - Mosaic: Provided multi-scale context, boosting performance on complex scenes.

- NMS and Confidence Threshold Tuning:

  - Lowering the confidence threshold from 0.5 to 0.4 improved recall of small traffic signs but introduced some extra false positives.

  - Adjusting NMS from 0.45 to around 0.50 pruned overlapping boxes but occasionally suppressed correct detections in crowded scenes.

- Anchor Box Tuning:

  - Enabling auto-anchor and manual anchor size adjustments yielded moderate gains in small sign localization accuracy, especially for signs below 32×32 pixels.

From these experiments, the final model showed consistently higher recall for small objects, with a slight trade-off in precision. Overall, it outperformed the baseline model at standard resolutions on key small-object metrics.

## 4. Suggestions for Improving Small Object Detection

- Continue High-Resolution Inputs:

  - Training at 1280×1280 (or even higher if resources allow) retains detail crucial for distant objects.

- Refine Data Augmentation:

  - Experiment with more advanced augmentations, such as Cutout or partial occlusion simulation, to handle hidden or partially visible signs.

- Focus on Anchor Box Optimization:

  - Auto-anchor is valuable, but further manual tuning might better match specific sign sizes in the dataset.

- Fine-Tune NMS/Confidence:

  - Explore adaptive thresholds or class-specific confidence levels to balance false positives vs. missed small objects.